# ESP32 Rain Sensor with Servo Control and ThingSpeak Integration

#include <ESP32Servo.h>
#include <WiFi.h>
#include <WiFiClient.h>
#include <ThingSpeak.h>

WiFiClient client;
Servo myServo;

// ===== WiFi & ThingSpeak Settings =====
const char* ssid = "vivo 1938";
const char* password = "123456789";
unsigned long myChannelNumber = 3048154;
const char* myWriteAPIKey = "SUSPXFZH6B7DZNI2";

// ===== Pin Assignments =====
const int rainSensorPin = 2;
const int rainAnalogPin = 34;
const int servoPin = 13;

// ===== Servo Settings =====
int stopPulse = 1500;
int forwardPulse = 1700;
int backwardPulse = 1300;

// ===== Movement Control =====
const int moveTime = 8000;

// ===== Rain Detection Threshold =====
const int rainThreshold = 1500;

// ===== State Tracking =====
bool isInside = false;

void setup() {
  Serial.begin(115200);

  pinMode(rainSensorPin, INPUT);
  myServo.attach(servoPin);

  myServo.writeMicroseconds(stopPulse);

  WiFi.begin(ssid, password);
  Serial.print("Connecting to WiFi");
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    Serial.print(".");
  }
  Serial.println("\nWiFi Connected!");

  ThingSpeak.begin(client);
}

void loop() {

  int digitalValue = digitalRead(rainSensorPin);
  int analogValue = analogRead(rainAnalogPin);

  Serial.print("Digital: ");
  if (digitalValue == LOW) {
    Serial.print("Raining (");
  } else {
    Serial.print("Dry (");
  }
  Serial.print(digitalValue);
  Serial.print(") | Analog: ");
  Serial.print(analogValue);

  bool isRaining = (analogValue < rainThreshold);
  if (isRaining) {
    Serial.println(" -> Wet (Rain Detected)");
  } else {
    Serial.println(" -> Dry");
  }

  if (isRaining && !isInside) {
    Serial.println("Rain Detected! Moving inside the shed...");
    ThingSpeak.setField(1, analogValue);
    ThingSpeak.writeFields(myChannelNumber, myWriteAPIKey);

    myServo.writeMicroseconds(forwardPulse);
    delay(moveTime);
    myServo.writeMicroseconds(stopPulse);
    Serial.println("Inside the shed and stopped.");
    
    isInside = true;
  }
  else if (!isRaining && isInside) {
    Serial.println("Rain stopped! Moving back out...");
    
    myServo.writeMicroseconds(backwardPulse);
    delay(moveTime);
    myServo.writeMicroseconds(stopPulse);
    Serial.println("Back out and stopped.");
    
    isInside = false;
  }

  delay(300);
}